# SCHEDULE “B”

PROJECT:	CASTLES OF KING CITY	 	CONTRACT NO.:

King City, Ontario

TRADE:	Heating and Air Conditioning		DATE:

SUB-CONTRACTOR TO SUPPLY and INSTALL ALL HEATING AND AIR CONDITIONING for the homes of this project, as described and defined on drawings and specifications prepared by RN Design, the Ontario Building Code, heating layouts and calculations and any by-laws of this municipality having jurisdiction and the plans and specifications listed within the contract documents, and marketing brochure, including but not limited to the following:

- All labour and material shall conform to the requirements of C.M.H.C., the National Association of Heating and Air Conditioning requirements, and the requirements of the municipality having jurisdiction.

- Prices, as noted, are based on architectural plans as provided.

- Supply and install approved LENNOX;

- Multi stage variable speed high efficiency gas furnace model SLP98V

- Humidifier model WB3-17

- Energy Star 2 stage Elite programmable thermostat

- Heat recovery ventilator model, HRV-200

- 14.5 seer air conditioner model 14ACX

- Case Coil model CX34

- Media Air Filter

Include all necessary connections, duct work, registers, grills, canvas connections, vibration dampers, thermostats and wall vents. Tape all duct work with foil tape. Caulk all penetrations through building envelop.

- Install hot water tank and venting as part of the agreement.

- Sub-Contractor has reviewed all engineering design layouts and heat loss/gain calculations for conformity to codes and design and construction efficiency.

- Prices include all permit and inspection fees.

- Supply and install all 1 ¼” gas piping from meter to furnace room. Provide gas line for BBQ connection at rear of home.  Provide gas line for stove in kitchen.

- Supply and install all ductwork only for bathroom and laundry room exhaust fans, as required, including all necessary insulation.

- Supply and install all necessary ducting and insulation for kitchen exhaust fans (6”) per O.B.C. Kitchen exhaust to be connected at time of hood installation. Install all ducting for bathroom and laundry room fans warped in insulation blanket as required. Supply and install all vent caps.

- Supply and install gas piping and venting for furnace and hot water tank. Cutting and patching holes through basement wall for gas pipe is included in this contract. All double venting of furnace and hot water tank must be vented to the exterior of the building envelope.  Main gas run to be sized for additional future gas appliances.  Provide for minimum four (4) additional T’s on gas line.  Includes all gas piping through garage and sleeving through porches as necessary minimum 1-1/4” diameter pipe.

- Cover all floor register openings to prevent accumulation of debris during construction.

- All rooms over garage spaces to include one supply run and one return air run installed in garage ceiling area below subfloor.

- Air conditioning units to be installed on wall brackets and secured to bracket. Includes permit and low and high voltage wiring.

- Sub-Contractor is not to cut or remove structural support members. It is this contractor’s responsibility to repair, brace, and support and have inspected any member (structural or otherwise) that has be damaged or cut by his forces.

- Finished basement areas to include all ductwork, return air run, register grills, exhaust vents and mechanical fans as required. Walkout basements are to be completed as “finished” spaces including low wall supply and return air.

- Sub-Contractor shall balance the heating system as required to ensure proper heating distribution.

- Supply all thermostats to Site Super for installation by others. Co-ordinate electrical work through Site Supervisor. Control wiring to furnace and Air Conditioning to be installed by this contractor.

- Supply operating manuals for all equipment to Owner’s Head Office on a per lot basis.

- Sub-Contractor must maintain a 24 Hour Emergency Service Department. The phone number and contact person must be attached to the furnace, and supplied to Owner upon contract award.

- Sub-Contractor to inspect all units and the work of previous trades to ensure proper installation and completion of work.  Sub-Contractor will advise the Site Superintendent, immediately, of any work in place which will adversely affect the work of this Contractor, or accept responsibility thereof.

- Sub-Contractor to remove all waste and scrap materials or packaging materials, which he or his forces may discard, from the project site.  A disposal bin will not be provided.  Failure of the Contractor to adhere to this stipulation will result in back charges.

- Sub-Contractor to schedule and co-ordinate all work and deliveries through the Site Superintendent. Check with Site Superintendent for any changes to plans and extra sheets.

- The Sub-Contractor should note that a Quality Control Program will be in place on this project, supervised directly by an independent technical auditor.  The Sub-Contractor may be required to make minor alterations in techniques and/or substitutions of materials and that these alterations/substitutions shall be included in this Contract.

- Provide two (2) year labour, material and workmanship warranty for all work as outlined herein for two (2) years from the date of occupancy.

- Sub-Contractor to inspect all units and the work of previous trades to ensure proper installation and completion of work.  Contractor will advise the Site Superintendent, immediately, of any work in place, which will adversely affect the work of this Contractor, or accept responsibility thereof.

- Sub-Contractor to remove all waste and scrap materials or packaging materials, which he or his forces may discard, from the project site.  A disposal bin will not be provided.  Failure of the Sub-Contractor to adhere to this stipulation will result in back charges.

- The Sub-Contractor should note that a Quality Control Program will be in place on this project, supervised directly by an independent technical auditor.  The Sub-Contractor may be required to make minor alterations in techniques and/or substitutions of materials and that these alterations/substitutions shall be included in this Contract.

- Provide two (2) year labour, material and workmanship warranty for all work as outlined herein for two (2) years from the date of occupancy.

- Sub-Contractor is responsible to execute and complete all deficiencies and service items in accordance with the minimum requirements of Tarion

- This Sub-Contractor agrees to complete outstanding service/deficiencies within one week after service call or work orders provided.

- Worker safety is of prime importance to Zancor Homes.  This sub-contractor abides to adhere and follow any and all safety requirements/procedures contained in main agreement and in accordance to the Ministry of Labour guidelines and regulations.

CONTRACTOR:						SUB-CONTRACTOR:

Zancor Homes (King City) Ltd.

137 Bowes Road

Concord, ON.  L4K 1H3

Tel. (905) 738-7010

Fax. (905) 738-5948

Per: __________________________			Per: _________________________

Authorized Signing Officer

Per: __________________________

Print Name: ___________________

Date:	________________________			Date: __________________________

We have the authority to bind the Corporation.